Slide 1: Title

Good afternoon, everyone. Today I’ll present a paper on timestamp manipulation attacks against the MAVLink 2.0 protocol for UAV communication.

Slide 2: Motivation

The core motivation is clear: MAVLink 2.0 added message signing to protect against forgery and replay, but it relies heavily on timestamps. And where do many drones get their time? From GPS — an unauthenticated, easily spoofed signal.

So the central question is: can an attacker break the protocol's security without recovering the secret key, simply by manipulating GPS time? The paper's answer, as we'll show, is yes.

Slide 3: Threat Model

Let’s start with the threat model established in the paper.

The attacker is assumed to have no access to any cryptographic key. But They can spoof GPS signals toward the drone, passively eavesdrop on MAVLink traffic, and inject packets over unencrypted links.

Critically, the drone accepts GPS as a time source and may lack a reliable internal real-time clock.

Our attack objectives are threefold: first, shift the drone's internal signing clock purely through GPS spoofing; second, exploit that shift to replay previously captured commands; and third, trigger a permanent denial of service by overflowing timestamp field.

Slide 4: Core Insight and Attack Logic

The paper's central insight is this: time itself becomes the weapon. The protocol uses a monotonically increasing timestamp to reject old messages. But if the drone's clock can be fast-forwarded into the future, then a legitimate message recorded during this spoof window can appear fresh when replayed later.

More strikingly, if the clock is pushed past its maximum representable value, the timestamp overflows, and suddenly all real world messages look like they come from the distant past — and are therefore rejected. The authors translate this insight into a multistage attack methodology, which I'll now walk through.

Slide 5: Attack Methodology

The first stage is GPS timestamp spoofing. The authors implemented a Python script that constructs forged GPS_INPUT messages with an arbitrary time offset — say, advancing time by one day or several months — and injects them over MAVLink. On a drone without a higher priority RTC, the flight controller accepts this time and updates its signing clock.

The second stage exploits this offset for replay attacks. The drone's clock is first spoofed far into the future. The ground station, to maintain communication, synchronizes to that spoofed time. A valid signed command — for example, a LAND instruction — is captured during this session. Later, after the drone is reset and relaunched with the same secret key, the captured packet is replayed. Because its timestamp is now in the future relative to the reset clock, the drone accepts it and executes the command.

The final and most severe stage is timestamp overflow. MAVLink uses a 48bit timestamp in 10 microsecond units, which can represent time up to roughly the year 2104. GPS time is spoofed to this maximum, causing the 48bit field to overflow and wrap to zero. However, the drone internally maintains a 64bit microsecond clock that continues counting forward. The result is a mismatch: every subsequent signed message carries a 48bit timestamp that is smaller than the internal 64bit time, so all messages are classified as stale. This produces a permanent, jamming style denial of service.

Slide 6: Experimental Validation

Now, let's turn to how the attacks were validated experimentally. The authors employed three test levels: software in the loop, hardware in the loop, and most importantly, full hardware tests with real telemetry radios and MAVLink signing enabled end to end.

Here is a summary of experimental results. During the rest of this talk, I'll focus on the hardware results, as they are the most definitive.

Just to add up by the way, during SITL and HITL, some important boundary conditions were discovered — for instance, USB connections can bypass signature checks, and an onboard RTC can block GPS time updates — but these were limitations of the test environment, not the attack itself. They actually motivated the push toward fully signed radio tests.

Slide 7: Key Hardware Results

Here are the critical findings from the full hardware experiments.

First, pure GPS based clock shifting without any key was demonstrated. When spoofed GPS_INPUT was sent over the air with a time one day in the future, the ground station observed an immediate jump of exactly 86,400 seconds in the `Received sig time` field. This confirmed that the UAV's signing clock directly tracks the injected GNSS time.

Second, the replay attack was validated. After pushing the drone's time forward, a signed LAND command was recorded. The signing state was then reset while keeping the same key, and that recorded packet was replayed. The drone accepted it, executed the landing, and advanced its internal clock to match the packet's future timestamp.

Third, the overflow DoS proved devastating. When GPS time was spoofed to the 48bit boundary, the timestamp overflowed, and immediately all incoming signed packets were rejected with bad signature errors. Even a previously valid future-dated command was ignored.

More critically, after this overflow, Q Ground Control could no longer connect even over a direct USB link, and reflashing the firmware via the ArduPilot bootloader could not clear the fault. The drone was left in a persistent, unrecoverable state.

Slide 8: Implications and Conclusion

To wrap up, what do these results tell us?

The MAVLink 2.0 protocol contains a protocol-level vulnerability: its replay protection relies on an unauthenticated external time source. An attacker without the secret key can weaponize GPS time to achieve replay attacks and permanent denial of service.

The broader lesson is that security cannot simply be bolted on by trusting external inputs. For resource-constrained systems like UAVs, secure timekeeping must be designed from within — for example, using time-since boot or authenticated time synchronization. This work demonstrates that leaving this trust in GPS opens a dangerous attack surface.